# EEEN30160 – Biomedical Signal Processing

# Assignment 1

Oisín Hogan

19355123

Contents

Problem 1:	3

Code:	3

Output	4

Discussion	4

Problem 2:	5

Code:	5

Outputs:	5

Discussion:	5

Problem 3:	6

Code:	6

Output:	6

Discussion	6

Problem 4:	7

Code:	7

Output:	7

Discussion:	8

Problem 5	9

Code:	9

Output: 	10

Discussion:	10

A	10

B	10

C	10

D	10

E	10

F	10

G	10

H	10

# Problem 1:

## Code:

function plotsinusoid2(f,a,startT,endT,step)

%plot a complex sinusoid of frequency, 'f' and amplitude, 'a', with 'startT'

%and 'endT' giving the bouns of the time and 'step' giving the step size

t = (startT:step:endT); % create array of timesf from start to end in the specified step size

compx = a*exp(2*pi()*f*t*i); % create a complex number using the specified inputs

Rex = real(compx);%get the real component of compx

Imx = imag(compx);%get the imaginary component of compx

Ax = (t*0)+a;% create an amplitude array that is the same size as t

tiledlayout(2,1)% create a 2 graph tiled layout to plot the angle and x

nexttile %go to the first tile

plot(t,Rex, 'r--') % plot the real part of x in red dashed line

hold on % keep the real part of x

plot(t,Imx, 'b-.'); % plot the imaginart part of x in a blue dot and dashed line

plot(t,Ax,'g-') % plot the amplitude of x

xlabel('time') % label x axis as time

ylabel('amplitude')% label y axis as amplitude

ylim ([-a-1 a+1]) % add 1 to the y bounds, as to increase the amount of graph shown

legend('Re(x)', 'Im(x)', 'Amp(x)')%add a legend

title('PlotSinusoid2 - Amplitude')%add a title

nexttile % next tile

plot(t,(360*f*t))% Plot the angle vs time

xlabel('time')%label x

ylabel('Angle')%label y

title('PlotSinusoid2 - Angle')% title

return

end

Figure 1

Output

## Discussion

Figure 2

The real and imaginary parts of the complex number are separated by a quarter step. The angle graph could be changed to reset to 360 each iteration using the modulo function, mod(a,x) , but this was not deemed necessary, when this was done, the graph looked like this:

# Problem 2:

## Code:

a = load('C:\Users\Oisin\Documents\MATLAB\Signal processing\EvokedPotential.mat'); % load file as 'a'

b = a.EvokedPotential;%let 'b' equal the evoked potential part of 'a'

l = size(b); % let 'l' equal the size of 'b'

t = [-100:650]; % let t range from -100 to 650 ms

plot(t, b)% plot 'b' against 't', each 1 on the x axis is equal to 1 ms

xlabel('time - ms')% label the x axis as time in ms

Figure 3

Outputs:

## Discussion:

The file was read into MATLAB and then plotted from the range of -100ms to 650ms, when this was done, the peak was found to be at 120ms, meaning that the subject was neurologically healthy.

# Problem 3:

## Code:

function ans = rmsfunct(n) % function to calculate the rms of a siganl

x = n.^2*sqrt(2);% let x be the square of each element in the array n, times root 2

N = size(n);% let N be the size of x

ans =sqrt(1/N(2)*sum(x.^2)); % ans equal ro the rms, done with the given function in the problem

end

n = [-728:0.5:1275]; % let n be an array from -728 to 1275 in steps of 0.5

rmsfunct(n)% calculate the rms of n

## Output:

ans =

8.4514e+05

## Discussion

The root mean square of this signal was 85140, when compared to the built in MATLAB function rms(), this number was confirmed

# Problem 4:

## Code:

struct = load('C:\Users\Oisin\Documents\MATLAB\Signal processing\EMG.mat');% load in EMG signal

vect = struct.EMG; %let 'vect' be the EMG signal

avect = abs(vect); %get absolute value of vect

plot(avect) %plot avect

lb = 4664;% lower bound observed from graph

ub = 11759;% upper bound observed from graph

avs =avect(lb:ub); %set the signal component of 'avect'

avnb = avect(1:lb); % set the lower bound of the noise component of 'avect'

avna= avect(ub:16001); % set the upper bound of the noise component of 'avect'

avn = [avnb; avna];% add them both together to get the total noise of 'avect'

ravn = rms(avn);% get rms of noise

ravs = rms(avs);% get rms of signal

SNR = 20*log(ravs/ravn) % use the rms values to get the SNR

Figure 4

Output:

SNR =

51.0796

## Discussion:

The signal frequency was Hz, and the noise frequency was Hz. The signal noise ratio was 51.0796.

# Problem 5

## Code:

struct = load('C:\Users\Oisin\Documents\MATLAB\Signal processing\vep.mat'); % read in the vep file

sigs = struct.vep;% get the signals from struct

sizes = size(sigs);% get the dimensions of sigs

Diff_Min_Max = max(sigs)- min(sigs); % find the difference between the min and max values of signal

avgdiff = mean(Diff_Min_Max);% find the mean difference of the signals

filteredvep = mean((sigs.'));% filtered vep is the mean of all the signals, .' transposes the matrix to ensure its the right dimensions

plot(sigs, '--')% plot all the signals in red dashed lines

hold on% hold all the signals

plot(filteredvep, 'K')%plot the filtered vep in black

ylabel('mV')% label the y axis as mV

maxv = max(filteredvep); % find the max of the filtered vep

minv = min(filteredvep); %find the min of the filtered vep

tmax = find(filteredvep == maxv);% find the time that the max occurs

tmin = find(filteredvep ==minv);% find the time that the min occurs

tdiff = abs(tmax - tmin);% find the difference in times

diffavg = maxv- minv;% find the average difference in the largest and smallest value

var20 = (std(sigs(1:20, :))).^2; % find the varience of the first 20 time steps of the signas

avgvar20 = mean(var20); % find the mean of these variences

varavg20 = (std(filteredvep(1:20)))^2; % vind the varience of the first 20 time steps of the filtered signal

Figure 5

Output:

## Discussion:

### A

Figure 5 has all the signals plotted together, all shown in different colour dotted lines

### B

The Diff_Min_Max was a 1 x 1263 array, where it was found by subtracting the max and min arrays from each other.

### C

The average value for Diff_Min_Max was found to be 24.230395622150300

### D

The filtered vep, or the average epoch is shown in Figure 5 in black.

### E

The max and min points of the vep occurred at 94 samples and 124 samples, with a time between occurrences of 30 samples.

### F

Because the average difference across all the trials looks at the extreme values, and the filtered signal has softened the large peaks of them, leading to a much smaller difference between the extremes.

### G

The variance of the different epochs for the first 20 frames was a 1263 x 1 array, with the average value of 10.439121218095861

### H

The variance of the first 20 samples of the filtered vep was 0.007396304600416, this was a lot lower than the average variance, as the extreme outliers that would have a large effect on the data were removed in the vep, leading to a much lower varience