# EEEN30160 – Biomedical Signal Processing

# Assignment 3

Oisín Hogan

19355123

Contents

Problem 1:	3

Code:	3

Output	4

Discussion	5

a)	5

b)	5

c)	5

Problem 2:	6

Code:	6

Outputs:	7

Discussion:	7

a)	7

b)	7

c)	7

d)	7

Problem 3:	8

Code:	8

Output:	9

Discussion	9

Problem 4:	10

Code:	10

Output:	11

Discussion	11

# Problem 1:

## Code:

clc

close all

clear

%clear all values, plots and command window

%a)

a = [1 .125 -1/2]; %observe a's from equation

b = [0, 2, 3]; % observe b's from equation

[z, p, k] = tf2zpk(b, a); %poles and zeros of system

zplane(z,p) %plot poles and zeros of system

title('Z-Plane')

%b)

%c)

x = rand(1000,1); % generate random vector x

xf = filtfilt(b, a, x); % filt x with filter

figure

tiledlayout(2,1)

nexttile% plot amplitude of unfiltered x

plot(x)

title('X( Unfiltered)')

xlabel('time')

ylabel('Amplitude')

nexttile%plot amplitude of filtered x

plot(xf, 'r')

title('X (Filtered)')

xlabel('time')

ylabel('Amplitude')

fdom = (0: 1/1000: 1-1/1000);% create frequency domain for x

fxf = abs(fft(xf)); % fourrier transform of filtered x

fxf = (fftshift(fxf)); % find phase of filtered x

fx = abs(fft(x)); % fourrier transform of x

fx = fftshift(fx);% find phase of x

figure

tiledlayout(2,1)

nexttile

plot(fdom, fx);

title('X')

xlabel('Frequency')

ylabel('Amplitude')

nexttile

plot(fdom, fxf)

title('X (Filtered)')

xlabel('Frequency')

ylabel('Amplitude')

Figure 1

Output

Figure 2

Figure 3

## Discussion

### a)

From observing the transfer function, the constants were found to be:

y(n) = 2x(n-1) – 3x(n-2) – 0.1250 y(n-1) + 0.5 y(n-2)

|  | A | B |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 0.125 | 2 |
| 2 | -0.5 | 3 |

Table 1

### b)

This system is not stable, as can be seen in the z-plane graph in figure 1, the poles lie outside the circle

### c)

This filter has amplified the signal x

# Problem 2:

## Code:

clc

close all

clear

%clear all values, plots and command window

%a)

N=100; % N = 100 for filter

A = [1, -.2, 0,  .1]; % define A according to transfer function

B = [1, .1, -.3]; % define B according to transfer function

[z, p, k] = tf2zpk(B, A); % find zeros and poles of transfer function

figure% plot the filters and poles of this function

zplane(z,p)

title('Z-Plane')

%b)

freqz(B, A, N);% plot the magnitude of hte filter response for the transfer function

title('Magnitude of Filter Response')

%c)

%d)

x = rand(1000,1);% random 1000 long signal

figure

tiledlayout(2,1)

nexttile% plot x

plot(x)

title('X signal')

xlabel('Samples')

ylabel('Amplitude')

nexttile % filter x and plot the filtered version

xf = filtfilt(B,A,x);

plot(xf)

title('Filtered X signal')

xlabel('Samples')

ylabel('Amplitude')

figure

tiledlayout(2,1)

nexttile

fdom = (0: 1/1000: 1-1/1000);% frequency domain array created

fxf = abs(fft(xf)); % fourrier transform of filtered x

fxf = (fftshift(fxf)); % phase shift of fft(filtered x)

fx = abs(fft(x)); % fourrier transform of x

fx = fftshift(fx);% phase shift of fft(x)

plot(fdom, fx); % plot phase shifted fft(x)

title('Filtered X signal')

xlabel('Frequencies')

ylabel('Amplitude')

nexttile

plot(fdom, fxf)% plot phase shifted fft(filtered x)

title('Filtered X signal')

xlabel('Frequencies')

ylabel('Amplitude')

Figure 4

Outputs:

Figure 5

Figure 6

Figure 7

## Discussion:

### a)

As can be seen in figure 4, all the zeros and poles lie inside the circle when plotted using the zplane() function. This meant that the filter was stable.

### b)

The plot for the frequency can be seen in figure 5.

### c)

This is a band pass filter, as can be seen in figure 5, where frequencies above and below about 0.35 of the normalised frequency were reduced.

### d)

From figure 7, it can be seen that the filter had no phase delay associated with it, which can be seen by the similarities of the 2 graphs.

# Problem 3:

## Code:

clc

close all

clear

%clear all values, plots and command window

%a)

fs = pi; % sampling frequency is pi Hz

Wn = 1/4; % the 1/8 pi is equal to 1/4 of pi/2

Nf = 12; % the order of the FIR filter is 12th

Bf = fir1(Nf, Wn); %Lowpass FIR filter

%b)

Nb = 3; % order of the butterworth filter is 3

[Bb, Ab] = butter(Nb, Wn); %Lowpass Buttereworth filter

%c)

figure

N= 100; % plot the fitler responses for N = 100

[Hf, Wf] = freqz(Bf, 1, N); % freqz for the FIR filter

[Hb, Wb] = freqz(Bb, Ab, N); % freqz for the butterworth filter

angf = angle(Hf); % find phase of Hf

angb = angle(Hb); % find phase of Hb

tiledlayout(2,1)

nexttile% plot the magnitude of Hf and Hb

fdom = (0: 1/100: 1-1/100);

plot(fdom , abs(Hf))

title('Response Magnitude of Filter')

xlabel('Normalised Frequency')

ylabel('Amplitude')

hold on

plot(fdom ,abs(Hb))

legend('FIR', 'Butterworth')

nexttile% plot the phase of Hf and Hb

plot(abs(angf))

title('Phase Response of Filter')

xlabel('Normalised Frequency')

ylabel('Amplitude')

hold on

plot(abs(angb))

legend('FIR', 'Butterworth')

## Output:

Figure 8

Discussion

From figure 8, it can be seen that the magnitude of both low pass filters reduces in similar ways, with a steeper initial and final decline found with the FIR filter in comparison to the Butterworth filter.

The phase response for both filters were very different, with the Butterworth filter once again being smoother.

# Problem 4:

## Code:

clc

close all

clear

%clear all values, plots and command window

T = 10;% time is 10s

fs = 300; % sampling frequency is 300Hz

tstep = 1/fs; %calculate the time step

t = (tstep:tstep : T); % create time array

f = (0.2: 0.2: 100); % create frequency array

A = 1./f; % create amplitude array

cossum = zeros(1, length(t)); % initialise cosine sum array

for j = 1:length(t) % for loop to create value for each time step

for i = 1:length(f) % for loop to sum all cosines together

cossum(j) = cossum(j) + A(i)*cos(f(i)*t(j)*2*pi); % add previous total to a*cosWt, where W is equal to 2*pi*f

end

end

fc = 10; %cut off frequency = 10 Hz

Wn = fc/(fs/2); %the cut off frequecny as a factor of fs/2

[B, A] = butter(3,Wn); % get filter constants

filtcos = filtfilt(B, A, cossum); % filter cossum

figure% plot of filtered vs unfiltered signal

plot(t, cossum, 'b.-')

hold on

plot(t, filtcos, 'r.-')

title('Signal vs filtered Signal')

ylabel('Value')

xlabel('Time (s)')

legend('Unfiltered', 'Filtered')

% calculate the amplitude and Phase of the original signal

ampcossum = abs(fft(cossum));

angcossum = fftshift(ampcossum);

%calculate the amplitude and phase of the filtered signal

ampfcos = abs(fft(filtcos));

angfcos = fftshift(ampfcos);

figure % plot the amplitudes

plot(ampcossum)

hold on

plot(ampfcos)

title('Amplitude of Signal vs filtered Signal')

ylabel('Amplitude')

xlabel('Time (s)')

legend('Unfiltered', 'Filtered')

figure% plot the Phases

plot(angcossum)

hold on

plot(angfcos)

title('Phase of Signal vs filtered Signal')

ylabel('Phase')

xlabel('Time (s)')

legend('Unfiltered', 'Filtered')

%calucalte the filter response

figure

freqz(B, A, 1000)

title('Magnitude of Filter Response')

Figure 9

Figure 10

Figure 11

Figure 12

Output:

Discussion:

The appropriate sampling frequency of 300 Hz was decided upon because the highest frequency cosine wave was 100 Hz. In order to prevent the occurrence of aliasing a sampling frequency of over twice this would be required, so 300 Hz was acceptable.

It can be seen from figure 9 that the filtering of the frequencies below 10 Hz, resulted in a lower peak at the high point of the function.