# EEEN30160 – Biomedical Signal Processing

# Assignment 4

Oisín Hogan

19355123

Contents

Problem 1:	3

Code:	3

Output	Error! Bookmark not defined.

Discussion	3

a)	Error! Bookmark not defined.

b)	Error! Bookmark not defined.

c)	Error! Bookmark not defined.

Problem 2:	4

Code:	4

Outputs:	5

Discussion:	6

a)	6

b)	Error! Bookmark not defined.

c)	Error! Bookmark not defined.

d)	Error! Bookmark not defined.

Problem 3:	6

Code:	7

Output:	9

Discussion	9

Problem 4:	Error! Bookmark not defined.

Code:	Error! Bookmark not defined.

Output:	Error! Bookmark not defined.

Discussion	Error! Bookmark not defined.

# Problem 1:

## Code:

clc

clear

close all% clear the command windows, workspace and figures

Fs = 2000;% set the sampling frequency

Fn = Fs/2;% set the nyquist

Ws = 100/Fn;Wp = 350/Fn;% calculate the Stop and Pass frequencies

Rp = 1; Rs = 40;% calculate the stop and pass ripples

%Calculate the orders and frequencies of the different filters

[Nb,Wcb]=buttord(Wp,Ws, Rp,Rs) %Butterworth filter

[Nc1,Wcc1]=cheb1ord(Wp,Ws,Rp,Rs) %Chebyshev Type-I

[Nc2,Wcc2]=cheb2ord(Wp,Ws,Rp,Rs) %Chebyshev Type-II

[Ne,Wce]=ellipord(Wp,Ws,Rp,Rs) %Elliptic

## Outputs

Nb =     4

Wcb =    0.2956

Nc1 =     3

Wcc1 =    0.3500

Nc2 =     3

Wcc2 =    0.1000

Ne = 3

Wce =    0.3500

## Discussion

### which filter would be better to use if minimizing the number of operations is one of the aims of your analysis?

Either the Elliptic, Chebyshev1 or Chebyshev2 filters would be the valid to minimise the number of operations as they had a lower exponent, 3,  than the Butterworth filter, which was 4.

### which filter would be better to use if a steep roll-off is a key requirement?

The elliptical filter would provide the steepest roll off

# Problem 2:

## Code:

clear

clc

close all% clear the command windows, workspace and figures

load("G:\My Drive\engineering_work\Biosigs\Lab 4\NoisyECG.mat")%Load the ECG file

Fs = 500;% sampling frequency

Fn = Fs/2;%Nyquist Frequency

Ampf = 500; % the amplification factor

Ecg = (ecg/500)*1000;% Find the original ecg signal in mV

len = length(ecg); % number of samples

%a)

tiledlayout(3,2)% Tiled Layout

nexttile

plot(Ecg) %plot original signal

xticks([]) % remove 'x' numbers, as they will be just on the bottom graphs

ylabel('Amplitude (mV)')% y labels

title('ECG Signal') % title

ecgf = abs(fft(Ecg));

ecgf = fftshift(ecgf);% fourrier transform of the signal

f = (-len/2:(len-1)/2)*Fs/len; %frequency range of the signal

nexttile

plot(f, ecgf)%plot the frequency domain of the signal

xticks([])

ylabel('Magnitude (dB)')

title('ECG Signal')

xlim([0, Fs/2])

%b)

Fc = 50;% choose frequency of notch

wo = Fc/Fn; % calcualte the adjusted frequency

bw = wo/30; % calculate bw

[b,a] = iirnotch(wo,bw); % get constants, 'a' and 'b'

ecg2 = filtfilt(b, a, Ecg); % Filter ecg

ecg2f = abs(fft(ecg2)); %get the frequencies of filtered ecg

ecg2f = fftshift(ecg2f);

nexttile

plot(ecg2)% plot filtered ecg in time and frequency domain

xticks([])

ylabel('Amplitude(mV)')

title('Filtered ECG Signal')

nexttile

plot(f, ecg2f)

xticks([])

ylabel('Magnitude (dB)')

xlim([0, Fs/2])

title('Filtered ECG Signal')

%c)

n = 5; % set order of butterworth filter

Fc = [2, 10]; % set pass band frequencies

Wn = Fc/Fn; % get the normalised pass band frequencies

[b,a]=butter(n,Wn); % filter constants

ecg3 = filtfilt(b, a, ecg2); % filter

nexttile

plot(ecg3) % plot final filtered signal in time and frequency domains

xlabel('Samples (1 Sample = 1/1000s)')

ylabel('Amplitude (mV)')

title('Filtered ECG Signal')

nexttile

ecg3f = abs(fft(ecg3));

ecg3f = fftshift(ecg3f);

plot(f, ecg3f)

xlabel('Frequency (Hz)')

ylabel('Magnitude (dB)')

title('Filtered ECG Signal')

xlim([0, Fs/2])

Figure 1

Outputs:

## Discussion:

### a)

The top left graph of Figure 1 show the noisy ecg signal in the time domain.

### b)

The top right graph of Figure 1 show the noisy ecg signal in the frequency domain.

### c)

Figure 2

The middle 2 graphs show the signal after the 50Hz interference was removed, figure 2 below is a zoomed in version of the frequencies for clarity, where it can be seen that the large peak at 50 Hz is not visible in the filtered signal

### c)

The movement artefact, seen as a large sine wave had a period of approximately 4 seconds, this meant that it had a frequency of about 0.25Hz, a filter was used to remove this artefact, and ony other frequencies lower than 2Hz, an upper limit of 10 Hz was selected to remove the noise that was seen in the frequency graphs. The lower 2 graphs in Figure 1 show the final filtered ecg signal in the time and frequency domains.

# Problem 3:

## Code:

clear

clc

close all% clear the command windows, workspace and figures

load("G:\My Drive\engineering_work\Biosigs\Lab 4\noisy_EMG.mat")% load the EMG file

Fs = 1000; % smapling frequency

Fn = Fs/2; % nyquist frequency

Fint = 50; % the frequency of interference (power line)

len = length(noisy_EMG); % number of samples

tiledlayout(5,2) % tiled layout

%1)

nexttile % plot the noisy emg in the frequency and time domain

plot(noisy_EMG)

title('Noisy EMG signal')

ylabel('Amplitude (mV)')

xticks([])

noisy_EMGf = abs(fft(noisy_EMG));

noisy_EMGf = fftshift(noisy_EMGf);

f = (-len:2:len-1)*Fn/len;

nexttile

plot(f, noisy_EMGf)

title('Noisy EMG Frequencies')

ylabel('Magnitude (dB)')

xticks([])

xlim([0, Fs])

%2)

Fc = Fint;% notch filter routine for power line interference at 50 Hz

wo = Fc/Fn;

bw = wo/30;

[b,a] = iirnotch(wo,bw);

middenemg = filtfilt(b, a, noisy_EMG);

nexttile % plot emg without 50 Hz interference in time and frequency domain

plot(middenemg)

xticks([])

title('Noisy EMG signal, w/o 50Hz interference')

ylabel('Amplitude (mV)')

nexttile

fmiddenemg = abs(fft(middenemg));

fmiddenemg = fftshift(fmiddenemg);

plot(f, fmiddenemg)

title('Noisy EMG w/o 50Hz interference Frequencies')

ylabel('Magnitude (dB)')

xticks([])

xlim([0, Fs])

n = 5; %choose order of butterworth filter

Fc = [20, 450]; % choose range of useful frequencies

Wn = Fc/Fn;% calculate the normalised frequency

[b,a]=butter(n,Wn, 'stop');  %butterworth band stop filter

ignorefreqs = filtfilt(b, a, middenemg); % filter the useful parts out

clean_emg = middenemg - ignorefreqs*(.7);%generate the clean emg by scaling and subtracting the ignored frequencies from the noisy emg

nexttile% plot clean emg in the time and frequency domains

plot(clean_emg)

xticks([])

title('Clean EMG Signal')

ylabel('Amplitude (mV)')

nexttile

fclean_emg = abs(fft(clean_emg));

fclean_emg = fftshift(fclean_emg);

plot(f, fclean_emg)

title('Clean EMG Frequencies')

ylabel('Magnitude (dB)')

xticks([])

xlim([0, Fs])

%3)

absemg = abs(clean_emg);%get the absolute value of the emg

nexttile% plot the absolute emg in the time and frequency domains

plot(absemg);

xticks([])

title('EMG Envelopes')

ylabel('Amplitude (mV)')

nexttile

fabsemg = abs(fft(absemg));

fabsemg = fftshift(fabsemg);

plot(f, fabsemg)

title('EMG Envelopes Frequencies')

ylabel('Magnitude (dB)')

xticks([])

xlim([0, Fs])

Fc = .8;%pick frequencies to filter the envelopes of the signal

Wn = Fc/Fn;%normalise

[b,a]=butter(n,Wn); %fitler constants

emgenv = filtfilt(b, a, absemg);%filter and obtain envelopes

%4) - 6)

gorlthresh = zeros(len,1);%initialise a zeros array for whether there is a signal or not

n = 3;%number of standard deviations from mean for cut off frequency

avg5 = mean(emgenv(1:Fs*5)); % find the mean of the first 5 seconds of just noise

sd5 = std(emgenv(1:Fs*5),1); % find the standard deviation of the first 5 seconds of noise

threshold = avg5+n*sd5; % Calculate the amplitude threshold = mean + n*standard deviation

for i = 1:len-1 %for loop to set the gorlthresh array equal to 1 if the signal is greater than the threshold

if emgenv(i)>threshold

gorlthresh(i) = 1;

end

end

nexttile%plot the envelopes in the time and freuqency domain, as well as whether or not the signal is active on the time domain

yyaxis left

plot(emgenv)

title('EMG Envelopes and Activation Signal')

xlabel('Time (samples)')

ylabel('Amplitude (mV)')

hold on

yyaxis right

ylabel('Activated (0/1)')

ylim([0, 1.1])

plot(gorlthresh)

nexttile

femgenv = abs(fft(emgenv));

femgenv = fftshift(femgenv);

plot(f, femgenv)

title('EMG Envelopes and Activation Signal Frequencies')

xlabel('Frequecny (Hz)')

ylabel('Magnitude')

xlim([0, Fc])

insts = find((diff(gorlthresh))>0)/Fs;

insts

Figure 3

Output:

insts =

7.7840

20.0160

22.4050

25.1010

27.7330

30.2350

32.6710

35.0160

37.5430

40.0350

42.7370

## Discussion

### 1)

The noisy signal can be seen plotted in the frequency and time domains in the top graphs of Figure 3.

### 2)

The 50Hz interference was filtered out in the same method as it was filtered in the previous question, with a notch filter. The remaining frequencies outside the 20 – 450 Hz range were filtered out on their own, and subtracted from the main signal, with a reduced value, in order to minimise their effect on the signal, while maintaining some of their data. The “clean” signal is plotted in the third row of Figure 3.

### 3)

The steps needed to extract the envelopes was to :

1.	Pre-process the signal – this was done in step 2

2.	Find the modulus of the signal – this was done using the abs() function

3.	Use a low pass filter on it – a cutoff frequency of 0.8 Hz was selected as it hade the signal adequately without noise

### 4)

The calculated mean for the noise of the first 5 seconds was 0.0373, and the standard deviation was 0.0044.

### 5)

After some trial and error, 2 standard deviations were found to be too sensitive, so 3 standard deviations were selected, this meant that there was a 97.5% that the noise values would not exceed this, value and that any big values that were recorded were not noise.

The bottom graphs in Figure 3 show the activations in red.

### 6)

The 11 activations were found using the diff() function in conjunction with the find() function, they can be seen below Figure3 above.