# EEEN30160 – Biomedical Signal Processing

# Final Project

Oisín Hogan

19355123

Contents

Background information	3

Pan-Thompkins Algorithm	3

Task 1 - Display	4

Output	4

Discussion	4

Task 2 - Filtering	5

Output	5

Discussion	5

Task 3- Segmentation	7

Output	7

Discussion	8

Task 4 and 5 – Classification of PVCs	8

Output	8

Discussion	8

QRS complex	9

R-R period	9

R-Wave Peak	9

Conclusion	9

References	9

Code	10

# Background information

## Pan-Thompkins Algorithm

The Pan-Thompkins algorithm is a method of identifying heart beats in an ECG signal. This method requires the use of:

- A band pass filter of frequency 5-15 Hz

- Derivation of the filter in order to highlight the QRS complex

- Squaring the signal

- Averaging of the signal to remove high frequency noise form it

- Thresholding the signal

This would be done in order to have a segmented signal that has singled out each heartbeat.

# Task 1 - Display

Figure 1

Output

Figure 2

## Discussion

The zoomed in graph of the ECG in figure 2 shows that there is a PVC in the wave that can be easily observed, on account of its large amplitude at its peak.

# Task 2 - Filtering

Figure 4

Figure 3

Output

## Discussion

From the top left plot of the ECG signal, a baseline wander of the signal can be seen, of approximately 1/250Hz, a cut off frequency of 1 Hz was selected as it would remove this artefact, this was done with a 3rd order Butterworth high pass filter. This effect can be seen from the change in the signal from the first 2 rows of figure 3.

Looking at the next pair of graphs, once the movement artefact had been removed, there was a lot of high frequency noise on the signal, this was removed with a 3rd order low pass filter, set for 30 Hz low pass. With this done, there was a filtered signal, that could be processed to identify PVCs. The effect of the low pass filtering can be seen in the second 2 rows of figure 4, where there is a zoomed in view of the ECG signal.

# Task 3- Segmentation

Figure 5

Output

Figure 7

Figure 6

## Discussion

In accordance with the Pan-Thompkins method, the signal had been pre-processed, the next step was to utilise a band pass filter at 5 – 15 Hz on the signal, this gave the top 2 graphs in figure 5.

Next the signal had to be differentiated, this was done using the MATLAB function diff(), this provided the first graph in figure 6.

After that the signal was squared, providing the blue line on the next graph of figure 6. An averaging filter of 0.15 seconds was used in order to smooth the signal. This is shown as the smooth, red line in the second graph in figure 6.

The smoothed data was used to find the instances of the ECG where there was a signal. A region of noise was identified between the 1.1 and 1.3 second interval, the mean and standard deviation of the noisy region was found here. This was used to provide the threshold of noise and signal, with experimentation, it was found that 6 standard deviations yielded 399 instances of this occurring which was acceptable.

A code was created to take each segment form the point that the instance was measured and add this to a ‘bundle’ array. This array was plotted, and the resulting graph is given in figure 7.

# Task 4 and 5 – Classification of PVCs

## Output

### S-Wave Analysis

| True Positive | True Negative | False Positive | False Negative | Accuracy |
| --- | --- | --- | --- | --- |
| 64 | 278 | 26 | 29 | 86.1460957178841 |

Table 1

### R-R wave interval

| True Positive | True Negative | False Positive | False Negative | Accuracy |
| --- | --- | --- | --- | --- |
| 56 | 275 | 29 | 37 | 83.3753148614610 |

Table 2

### R–Peak Amplitude

| True Positive | True Negative | False Positive | False Negative | Accuracy |
| --- | --- | --- | --- | --- |
| 66 | 271 | 33 | 27 | 84.8866498740554 |

Table 3

## Discussion

The 2 methods of identifying the PVCs that were used for the purposes of this were the abnormal QRS complex shape and the compensatory R-R pause, that was associated with the PVC.

### QRS complex

Figure 8

From observing figure 7 above, the shape of the PVC S wave can be seen to be either 1 of 2 distinct shapes, where there is a sharp dip, with a rapid rise, and one where there is a gradual rise. Figure 8 below shows that a point was selected between the 2 shapes, this point would be used to classify the difference between the 2 shapes and select the PVCs.

Table 1 above in the results shows the effect that classifying by the S wave was found to have an accuracy of over 86%.

### R-R period

The length of each line of the array ‘bundles’ was counted, and that number was used to classify the length of the epoch. This method of classification yielded the results seen in table 2 above.

## R-Wave Peak

Figure 9

When a PVC occurs at a contraction, this can result in an increased amplitude of the R-peak. For this method of classification, a cut off amplitude of 700 mV was selected, and used to drive the classification system, the results of which can be seen in table 3 above.

# Conclusion

The 3 methods of identifying PVCs were all of similar efficacy, having an accuracy of 85±2%. The most accurate method of identifying them that was examined in this report was the S-wave analysis which differentiated between each wave on account of the shape that the s- wave of the epoch made. Increased accuracy could possibly be achieved using a Support Vector Machine classifier to combine the different features in order to more accurately classify PVCs.

# References

5-3. lesson 5 (cont) ventricular arrhythmias (no date) ECG Learning Center - An introduction to clinical electrocardiography. Available at: https://ecg.utah.edu/lesson/5-3 (Accessed: December 20, 2022).

Sedghamiz, H. (2014) MATLAB implementation of Pan Tompkins ECG QRS detector. - researchgate, Matlab Implementation of Pan Tompkins ECG QRS Detector. Available at: https://www.researchgate.net/profile/Hooman-Sedghamiz/publication/313673153_Matlab_Implementation_of_Pan_Tompkins_ECG_QRS_detector/data/5ac701130f7e9bcd51932f19/Sedghamiz-2014-Pan-Tompkins-Matlab.pdf (Accessed: December 20, 2022).

Wang, J. et al. (2021) Lead I R-wave amplitude to distinguish ventricular arrhythmias with lead V3 transition originating from the left versus right ventricular outflow tract, Clinical cardiology. U.S. National Library of Medicine. Available at: https://www.ncbi.nlm.nih.gov/pmc/articles/PMC7803356/ (Accessed: December 20, 2022).

# Code

clc

clear

close all% clear the command windows, workspace and figures

load("G:\My Drive\engineering_work\Biosigs\Project\ecgproject.mat")% load in ecg file

%Part 1

fs = 200;

fn = fs/2;

N = length(ecg1);

t = (0:1:N-1)/fs;% establish constants for the sampling and nyquist frequencies, and the time domain of the signal

figure()

plot(t, ecg1)

title('Plot of ECG Signal in Time Domain')

xlabel('Time(s)')

ylabel('Amplitude(mV)')

%xlim([1,2])

%part 2 - filtering

ecgang1 = fft(ecg1);

ecgang1 = fftshift(abs(ecgang1));

f = (-N/2:1:N/2-1)/N;

f = f*fs;% create frequency domain array and a fourrier transform of the ECG signal to the frequency domain

figure()% create a new figure for the output of task 2

tiledlayout(3,2)

nexttile

plot(t, ecg1)

title('Plot of ECG Signal in Time Domain')

xlabel('Time(s)')

ylabel('Amplitude(mV)')

%xlim([1,2])

nexttile

plot(f, ecgang1)

title('Plot of ECG Signal in Frequency Domain')

xlabel('Frequency (Hz)')

ylabel('Magnitude(dB)')

xlim([0, fs/2])

fcolow = 1;% choose high pass cut off frequency

wh = fcolow/fn; %] calcualte the adjusted frequency

[b,a] = butter(3, wh, 'high'); % get constants, 'a' and 'b'

ecg2 = filtfilt(b, a, ecg1); % Filter ecg

ecgang2 = abs(fft(ecg2)); %get the frequencies of filtered ecg

ecgang2 = fftshift(ecgang2);

nexttile

plot(t, ecg2)

title('Plot of ECG Signal in Time Domain')

xlabel('Time(s)')

ylabel('Amplitude(mV)')

%xlim([1,2])

nexttile

plot(f, ecgang2)

title('Plot of ECG Signal in Frequency Domain')

xlabel('Frequency (Hz)')

ylabel('Magnitude(dB)')

xlim([0, .5*fs]) %plot the filtered ecg in the time and frequency domains

fcohigh  = 30; %set the frequency for the lowpass filter

wl = fcohigh/fn;

[b,a] = butter(3, wl);

ecg3 = filtfilt(b,a, ecg2);

ecgang3 = fft(ecg3);

ecgang3 = fftshift(abs(ecgang3)); % filter the ecg and obtain the fourier transform of the signal

nexttile

plot(t, ecg3)

title('Plot of ECG Signal in Time Domain')

xlabel('Time(s)')

ylabel('Amplitude(mV)')

%xlim([1,2])

nexttile

plot(f, ecgang3)

title('Plot of ECG Signal in Frequency Domain')

xlabel('Frequency (Hz)')

ylabel('Magnitude(dB)')

xlim([0, fs/2]) % plot final filtered signal in the time and frequency domain

%Part 3 - Segmentation

fband = [5, 15]/fn; % for the first part of the PT method, the filter pass band was established

[b,a] = butter(3, fband);

ecg4 = filtfilt(b, a, ecg3);% filter ecg with the passband frequencies

figure()

tiledlayout(2,1)

nexttile

plot(t, ecg4)

title('PT Method Part 1')

xlabel('Time(s)')

ylabel('Amplitude(mV)')

nexttile

plot(t, ecg4)

xlim([1,2])

title('PT Method Part 1 (Zoomed in)')

xlabel('Time(s)')

ylabel('Amplitude(mV)')% create a new figure to plot the filtered signal as a whole and zoomed in

ecg5 = diff(ecg4);

dt = t(1:N-1); % differentiate the filtered signal

figure()

tiledlayout(2, 1)

nexttile

plot(dt, ecg5)

title('PT Method Part 2 (Zoomed in)')

xlabel('Time(s)')

ylabel('Amplitude(mV/s)')

xlim([1,2])%plot zoomed in differentiated signal

nexttile

ecg6 = ecg5.^2;% square signal

plot(dt, ecg6)

title('PT Method Part 3 ,4 & 5 (Zoomed in)')

xlabel('Time(s)')

ylabel('Amplitude(mV/s ^2)')

xlim([1,2]) % plot zoomed in squared signal

hold on

fsa = round(fs*.15); % set adjusted frequency for averaging each .15 seconds of the ecg

coeffavg = ones(1, fsa)/fsa; % create 'b' coefficient for the filtering of the signal

ecg7 = filtfilt(coeffavg, 1, ecg6); %filter the signal

plot(dt, ecg7)% plot the filtered signal over the other signal

noisesig = ecg7(1.1*fs:1:1.3*fs); % create an array of the signal where there is only noise

threshmean = mean(noisesig); % find the mean of the noise

threshsd = std(noisesig);% find the standard deviation of the noise

thresh = threshmean+6*threshsd; % experimentally it was foudnd that 6 standard deviations of the mean yielded 399 intances of signals being caught

%this was the threshold of amplitude

gorl = zeros(1, N-1);%create an array to see if the threshold has been passed or not

for i = (1:N-2)

if ecg7(i)>thresh

gorl(i) = 1;

end

end % for loop and if statement to fill out gorl array

insts = find((diff(gorl))>0);% find the intances where the threshold is passed, in samples

plot(dt, gorl*10^4) % plot this data on the same graph as the parts 3 and 4

n = length(insts);

bw = 300;

bundle = zeros(bw, n-1);

%bundle = zeros(bw, n);

for j = 1:n-1

for i = 1:bw

if i +insts(j)<insts(j+1)

bundle(i, j) = ecg3(i +insts(j));

else

break

end

end

end

figure()

plot(bundle)

title('Bundles of heart beats')

xlabel('Samples')

ylabel('Amplitude (mV)') %Plot each epoch on the same axis in order to identify differences between the PVC and normal contraction

%step 4 and 5 - Classification

%method 1- S wave analysis

compsampno = 74; %select time for amplitude to be measured on

ampthresh = -140; % set cut off amplitude

tfswave = zeros(n-2, 1); %create a 0's array for results

for i = 1:n-2

if bundle(compsampno, i)<ampthresh

tfswave(i) = 1;

end

end % for loop to generate answer set

% compare results

tp = 0;

tn = 0;

fp = 0;

fn = 0; % initialise the final answer variables

for i = 1:length(tfswave)

if tfswave(i)== 1&& isPVC(i) ==1

tp = tp+1;

end

if tfswave(i)== 1&& isPVC(i) ==0

fp = fp+1;

end

if tfswave(i)== 0&& isPVC(i) ==0

tn = tn+1;

end

if tfswave(i)== 0&& isPVC(i) ==1

fn = fn+1;

end

end %work out the amount of true and false positive and negative answers

acc = (tp+tn)*100/ (tp+tn+fp+fn); % calcualte accuracy

resultssw = {'True Positive', 'True Negative', 'False Positive', 'False Negative', 'Accuracy' ;tp, tn, fp, fn, acc};% create a dataframe of the results of this method

%method 2 R-R length

rrlegth = 140;% select a cut off length of 140 samples

tfrlen = zeros(n-2, 1);

for i = 1:n-2

len = max(find(bundle(:, i)));% calculate the length of the line in question

line = bundle(1:len, i); %take out the nonzero numbers from the line

if length(line)>rrlegth

tfrlen(i) = 1;

end

end

% compare results

tp = 0;

tn = 0;

fp = 0;

fn = 0;

for i = 1:length(tfrlen)

if tfrlen(i)== 1&& isPVC(i) ==1

tp = tp+1;

end

if tfrlen(i)== 1&& isPVC(i) ==0

fp = fp+1;

end

if tfrlen(i)== 0&& isPVC(i) ==0

tn = tn+1;

end

if tfrlen(i)== 0&& isPVC(i) ==1

fn = fn+1;

end

end

acc = (tp+tn)*100/ (tp+tn+fp+fn);

resultsrl = {'True Positive', 'True Negative', 'False Positive', 'False Negative', 'Accuracy' ;tp, tn, fp, fn, acc};% fill out results array in the same way as in method 1

%method 3- R Peak Amplitude Analysis

ampthresh = 700;% set an amplitude threshold of 700 mV

rpawave = zeros(n-2, 1);

for i = 1:n-2

maxamp = max(bundle(:, i));% find the maximum peak value in each epoch

if maxamp>ampthresh

rpawave(i) = 1;

end

end

% compare results

tp = 0;

tn = 0;

fp = 0;

fn = 0;

for i = 1:length(rpawave)

if rpawave(i)== 1&& isPVC(i) ==1

tp = tp+1;

end

if rpawave(i)== 1&& isPVC(i) ==0

fp = fp+1;

end

if rpawave(i)== 0&& isPVC(i) ==0

tn = tn+1;

end

if rpawave(i)== 0&& isPVC(i) ==1

fn = fn+1;

end

end

acc = (tp+tn)*100/ (tp+tn+fp+fn);

resultsra = {'True Positive', 'True Negative', 'False Positive', 'False Negative', 'Accuracy' ;tp, tn, fp, fn, acc};%cacluate results in the same method as method 1 and 2